# METHOD OF STATEMENT

Date: 14th Aug 2025

Description of work

To shift out 4 units train from workshop to loading area, hoist up onto lorry chassis and deliver to Eric Wong warehouse for packing

Machine Spec

4 units Train Bogie

Dim: 270 x 225 x 100 cm each

Wt: 5, 000 kg each

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

#### SMRT Depot @ Bishan

Equipment Use

2 x 633 lorry loader

Low loader

Hydraulic/hand jack

Woodblock

Rollers/pallet truck

Wire sling

Shackle

Obstacles

No obstacle on operation area and route to machine position

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting area

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Jack up train to 4” high and let it sit on solid woodblock

- Insert rollers under train bed

- Shift out train from workshop to loading area

- Riggers to attached required sling to crane hook

- Secure sling onto rigging point of train

- Once rigging work is done, signalman to give signal to crane operator to hoist up train from the ground

- Load up train onto low loader chassis

- Secured train with strapping belt prior to moving out of customer premise

- Transport and deliver to Eric Wong warehouse for packing and load onto container

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175