# METHOD OF STATEMENT

Date: 25th Jan 2024

Description of work

To shift out 1 unit machine and accessories from production area to loading bay, hoist and load up onto low-loader, lorry chassis, secured, and deliver to another location

Machine Spec

1 unit Nigata machine

Dim: 5.73 x 3.3 x 3.93

Wt: 20, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

31/01/24  -  to shift out machine accessories and load up onto lorry chassis

01/02/24  -  to shift out machine and load up onto low-loader

Time of Operation

09:30 am

Location of Operation

#### 1 Seletar Aerospace Crescent

#### Rolls-Royce Singapore Pte Ltd

Equipment Use

1 x 110ton lorry crane

633 lorry loader

1 x low-loader

1 x forklift

Rollers/pallet truck

Wire/webbing sling

Shackle

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacle on operation area and route to machine position

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to guide and control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting area

- To shift out machine and accessories from production to loading area

- Riggers to attached required lifting gears to crane hook

- Secure sling to rigging point of machine

- Once rigging work is done, signalman to give signal to crane operator to hoist up machine

- To reverse low-loader to near machine

- Unload machine onto low-loader chassis

- Release all sling from rigging point of machine

- Secured machine with belt prior to being transport to another location

- Using of forklift to load up machine accessories onto lorry chassis

- Secured with strapping belt prior to being transport to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175