# METHOD OF STATEMENT

Date: 06th July 2023

Description of work

To shift out 1 unit machine to loading area, hoist and load up onto customer forwarder lorry

Machine Spec

1 unit Vacuum Coating machine

Dim: 230 x 170 x 228 cm

Wt: 3, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

15/07/23

Time of Operation

09:30 am

Location of Operation

#### 18 Woodlands Loop

Edmund Optics (S) Pte Ltd

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire sling

Canvas/metal plate (to lay out for floor protection)

Obstacles

Ensure no obstacles around operation area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in place                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine using crowbar and placed woodblock under machine bed

- Insert pallet truck to machine bed

- Shift out machine from 4th floor via cargo lift

- Transport machine to loading bay

- Insert sling to carne hook

- Secured sling to machine rigging point

- Lorry loader to hoist up machine

- Load up machine onto forwarder lorry

- Forwarder to secured machine with strapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175