# METHOD OF STATEMENT

Date: 14th July 2022

Description of work

To shift out 1 unit press machine from production area to loading bay, hoist up onto lorry chassis and deliver to new location

Machine Spec

1 unit Kyori 60ton Hi Speed Press

Dim: 240 x 160 x 270 cm

Wt: 11, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

19/07/22

Time of Operation

09:45 am

Location of Operation

#### 20 Benoi Sector

#### Shimano (S) Pte Ltd

Equipment Use

1 x 633 lorry loader

Lorry loader

Rollers/pallet truck

1 x platform

Wire/webbing sling

Shackle

Obstacles

No obstacle on operation area and route to machine position

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to guide and control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated hoisting area

- Shift out machine from production to loading area

- Attached required lifting gears to crane hook and secure to rigging point of machine

- Lorry crane to hoist up machine

- Load up machine onto lorry chassis

- Release all sling from crane hook

- Secured machine with strapping belt

- Transport machine to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175