# METHOD OF STATEMENT

Date: 23rd Oct 2025

Description of work

To shift out 1 unit machine from production to loading area, hoist and load up onto lorry chassis

Machine Spec

1 unit Sunnen HTE-1600 machine

Wt: 1, 200 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

24/10/25

Time of Operation

09:00 am

Location of Operation

#### 273 Jalan Ahmad Ibrahim

#### Baker Hughes Singapore Pte

Equipment Use

1 x lorry loader

Crowbar

Woodblock

Rollers/pallet truck

Wire/webbing sling

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacle on operation area and route to machine position

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated hoisting area

- Setting up of crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Lift up machine to 4” high and let it sit on solid woodblock

- Insert rollers/pallet truck under machine bed

- Shift out machine from production to loading area

- Riggers to attached required sling to lorry crane hook and secure to rigging point of machine

- Hoist and load up machine onto lorry chassis

- Secure with strapping belt prior to moving out of customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175