# METHOD OF STATEMENT

Date: 20th Oct 2023

Description of work

To shift out 1 unit machine from 7th floor via cargo lift to loading area, hoist up onto lorry chassis and deliver to another location

Machine Spec

Milltap 700

Dim: 300 x 230 x 285 cm

Wt: 4, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

30/10/23

Time of Operation

09:45 am

Location of Operation

#### 4 Fusionpolis Way

#### Kinesis Building

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire sling

Crowbar/woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

Any debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Barricade off operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Shift out machine from 7th floor to loading area via cargo lift

- Rigger to attached required sling to crane hook

- Secured sling    to rigging point of machine

- Hoist up machine and load onto lorry chassis

- Release all sling from rigging point of machine

- Secured machine with strapping belt

- To transport and deliver machine to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175