# METHOD OF STATEMENT

Date: 05th Mar 2024

Description of work

To shift out 2 unit machine from production to loading area, hoist up onto lorry chassis and deliver to another location

Machine Spec

1 unit HAAS TL25 CNC

Dim: 360 x 185 x 190 cm / Wt: 7, 000 kg

1 unit DOOSAN LYNX220B Turning Center

Dim: 210 x 160 x 170 cm / Wt: 2, 900 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

07/03/24

Time of Operation

10:30 am

Location of Operation

#### ST Engineering Aerospace Systems Pte Ltd

#### Collection: 51 Loyang Dr

#### Deliver: 56 Loyang Way

Equipment Use

2 x 633 lorry crane

Rollers/pallet truck

Wire/webbing sling

Shackle

Crowbar

Hydraulic/hand jack

Woodblock

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in place                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry loader at designated location for the hoisting work

- Set up crane with outriggers fully extended

- Jack up machine and insert rollers under machine bed

- Shift out machine from production

- Transport machine to loading area

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine

- Hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up machine and onto lorry chassis

- Secured machine with strapping belt prior to moving out of customer premise

- Transport and deliver machine to another factory

- At new factory, hoist down machine from lorry chassis

- Insert rollers/pallet truck under machine bed

- Shift and transport into factory premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175