# METHOD OF STATEMENT

Date: 08th Aug 2024

Description of work

To shift out machines and accessories from production area to loading bay, hoist and load up onto lorry chassis and deliver to another location

Machine Spec

1 unit MNH5 Makino A55

Dim: 360 x 240 x 285 cm / Wt: 8, 000 kg

1 unit 4NE-600 CNC Lathe

1 unit TN01 Hitachi Seiki CNC Lathe

Dim: 350 x 220 x 190 cm / Wt: 5, 000 kg

1 unit TN16 Hitachi Seiki 5NE CNC Lathe

Dim: 390 x 220 x 190 cm / Wt: 5, 000 kg

1 unit GR104 Okamoto CG

Dim: 240 x 140 x 190 cm

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

17/08/24

Time of Operation

09:40 am

Location of Operation

#### 7 Joo Koon Circle

#### Honeywell

Equipment Use

633 lorry loader

1 x forklift

Rollers

Pallet truck

Wire/webbing sling

Shackle

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacle on operation area and route to machine position

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting area

- Set up crane with outriggers fully extended and resting on timber mats as base plate

- Jack up machine to 3” high

- Insert rollers/pallet truck under machine bed

- Shift out machine from production to loading area

- Riggers to attached required lifting gears to crane hook

- Secure sling to rigging point of machine

- Once rigging work is done, signalman to give signal to crane operator to hoist up machine

- Load up machine onto lorry chassis

- Release all sling from rigging point of machine

- Secured machine with strapping belt prior to being transport out of customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175