# METHOD OF STATEMENT

Date: 04th July 2024

Description of work

To shift out 2 unit machine and accessories from production area to loading bay, hoist and load up onto low-loader/lorry chassis and deliver to 2 different location

Machine Spec

1 unit Kafo VMC-168

Wt: 16, 000 kg

1 unit Kafo 1688

Wt: 20, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

04/07/24

Time of Operation

09:30 am

Location of Operation

#### Enwin Engineering Pte Ltd

Move out address – 77 Pandan Loop

#### Move in address – 97 Pandan Loop (Kafo VCM-168)

Move in address – 51 Pandan Loop (Kafo 1688)

Equipment Use

1 x 50ton mobile crane

633 lorry loader

1 x low-loader

1 x forklift

Rollers/pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on operation area and route to machine position

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to guide and control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting area

- Set up crane with outrigger fully extended and resting on steel plate/timber mats as base plate

- To shift out machine from production to loading area

- Riggers to attached required lifting gears to crane hook

- Secure sling to rigging point of machine

- Once rigging work is done, signalman to give signal to crane operator to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up machine

- To reverse low-loader to near machine

- Unload machine onto low-loader chassis

- Release all sling from rigging point of machine

- Transport to another factory

- At new factory, lorry loader to hoist down machine from low-loader

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and position into factory premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175