# METHOD OF STATEMENT

Reference No: EW/Q/1061/23

Date: 02nd Aug 2023

Description of work

To shift out 8 unit machine from workshop to loading area, hoist up onto lorry chassis and deliver to another location

Machine Spec

Refer to attached machine list

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

07/08/23

Time of Operation

09:40 am

Location of Operation

#### 8 Somapah Rd

SUTD, Blk 5 #01

Equipment Use

2 x 633 lorry crane

Rollers/pallet truck

Wire/webbing sling

Crowbar/hand jack

Woodblock

Shackle

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in place                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Set up crane with outriggers fully extended

- Jack up machine and placed woodblock under machine bed

- Insert pallet truck to below machine

- Shift out machine from workshop

- Transport machine to loading bay

- Insert sling to crane hook and secured to rigging point of machine

- Lorry loader to hoist up machine

- Load up machine onto lorry chassis

- Release all sling from machine rigging point

- Secured machine with strapping belt prior to moving out of premise

- Transport and deliver to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175