# METHOD OF STATEMENT

Date: 17th July 2023

Description of work

To shift out 3 unit Injection Moulding machine from production to loading area, hoist and load up onto lorry chassis

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

22/07/23 (Saturday)

Time of Operation

09:30 am

Location of Operation

#### Blk 514 Chai Chee Lane #01-04

Design Plastech Pte Ltd

Equipment Use

3-4 x lorry crane

Rollers

Pallet truck

Wire sling

Woodblock

Hand jack

Obstacles

Ensure no obstacles around operation area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in place                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine using hand jack

- Insert rollers to machine bed

- Shift out machine from production area

- Transport machine to loading bay

- Insert sling to crane hook and secured to rigging point of machine

- Lorry crane to hoist up machine

- Load up machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of tenant premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will clear and disposed

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175