# METHOD OF STATEMENT

Date: 14th Mar 2025

Description of work

To shift out 1 unit machine and accessories from workshop to loading area, lorry crane to hoist up and load onto lorry chassis

Machine Spec

Mill E500U

Dim: 2650 x 3300 x 2450 mm

Wt: 6, 030 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

27-28/03/25

Time of Operation

09:30 am

Location of Operation

Singapore Institute of Technology

#### 10 Dover Dr

Equipment Use

2 x 633 lorry crane

Hydraulic/hand jack

Rollers/pallet truck

Woodblock

Wire/webbing sling

2 pcs 4 x 8ft metal plate (to lay out for uneven ground)

PVC sheet/canvas (to lay out for floor protection)

Obstacles

No obstacle on crane parking

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated location

- Set up crane with outrigger fully extended and resting on steel plate/timber mats as base plate

- To jack up machine to 4” high

- Insert rollers under machine bed

- Shift out machine from production to loading area

- Riggers to attached required sling onto crane hook

- Secure sling to rigging point of machine

- Hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist and load up machine onto lorry chassis

- Released all sling from rigging point of machine

- Secured machine with strapping belt prior to moving out of customer premise

- Transport and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175