# METHOD OF STATEMENT

Date: 05th Dec 2022

Description of work

To shift out 5 unit machine from production to loading area, hoist up onto lorry chassis and deliver to another location

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

12/12/22 (Monday)

Time of Operation

09:45 am

Location of Operation

#### Blk 115A Commonwealth Drive #01-1/02

Acot Precision Engineering Pte Ltd

Equipment Use

2-3 x 633 lorry loader

Rollers/pallet truck

Wire sling

1 x platform (to lay out for machine pathway)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Unload and lay out platform directly in front of tenant door for machine pathway

- Shift out machine from production room onto platform

- Insert sling to crane hook and secure onto machine hoisting point

- Lorry loader to hoist up machine

- Load machine onto lorry chassis

- Release all sling from crane hook

- Secured machine with strapping belt prior to moving out of customer premise

- Transport and deliver machine to Eric Wong warehouse

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175