# METHOD OF STATEMENT

Date: 02nd Sep 2022

Description of work

To shift out 1 unit machine from production area to loading bay, hoist up and dismantle machine into 2 part, load onto U-bed, secured, and deliver to another location

Machine Spec

1 unit VTL2000 ATC Machine (to dismantle to 2 parts)

Top part

Wt: 28, 900 kg

Below part

Wt: 22, 810 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

09/09/22 (to break cement around machine leg and to jack up machine to standby for tomorrow moving)

10/09/22 (shift out machine to loading bay and hoist up onto U-bed trailer)

Time of Operation

09:45 am

Location of Operation

#### 149 Gul Circle

#### FMC Technologies Singapore Pte Ltd

Equipment Use

1 x 70ton mobile crane

1 x U-bed

1 x low-bed

1 x 633 lorry loader

1 x forklift

Rollers/pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on operation area and route to machine position

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to guide and control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile crane at designated hoisting area

- Shift out machine from workshop to loading area

- To assist customer engineer to dismantle machine to 2 part

- Riggers to attached all required lifting gears to crane hook and secure onto rigging point of machine

- Once rigging work is done, signalman to give signal to crane operator to hoist up machine

- Reverse U-bed to near machine

- Load machine onto U-bed chassis

- Release all sling from rigging point of machine

- Secured machine with belt prior to being transport to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175