# METHOD OF STATEMENT

Date: 19th Nov 2024

Description of work

To shift out 2 unit machine and accessories from production area to loading bay, hoist up onto low loader/lorry chassis and deliver to new location

Machine Spec

1 unit Mazak i700

Dim: 410 x 240 x 310 cm

Wt: 16, 100 kg

1 unit Mazak VCN530

Dim: 319 x 308 x 304 cm

Wt: 6, 800 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

21/11/24

Time of Operation

10:00 am

Location of Operation

#### 23 Changi North Crescent

UMS Pte Ltd

Equipment Use

1 x 75ton lorry crane

633 lorry crane

1 x low loader

1 x forklift

Hydraulic/hand jack

Electric roller/rollers

Wire/webbing sling

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated location

- Set up crane with outrigger fully extended and resting on steel plate/timber mats as base plate

- To jack up machine and insert rollers

- Shift out machine from production to loading area

- Riggers to attached sling to crane hook

- Secure sling to rigging point of machine

- Hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist and load up machine onto low loader/lorry chassis

- Released all sling from rigging point of machine

- Secured machine with scrapping belt

- Transport machine out from customer premise

- Deliver to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175