# METHOD OF STATEMENT

Date: 11th Oct 2022

Description of work

To shift out 1 unit machine from production area to loading bay, hoist up onto lorry chassis and deliver to another location

Machine Spec

1 unit Weiler E90 CNC Lathe

Dim: 300 x 200 x 220 cm

Wt: 8, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

12/10/22

Time of Operation

09:45 am

Location of Operation

#### 273 Jalan Ahmad Ibrahim

#### Baker Hughes Singapore Pte

Equipment Use

1-2 x 633 lorry loader

Rollers/pallet truck

Hand jack

Solid woodblock

Wire/webbing sling

Obstacles

No obstacle on operation area and route to machine position

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry loader at designated hoisting area

- Shift out machine from workshop to loading area

- Riggers to attached required lifting gears to crane hook and secure onto rigging point of machine

- Once rigging work is done, signalman to give signal to crane operator to hoist up machine

- Load up machine onto lorry chassis

- Release all sling from rigging point of machine

- Secured machine with strapping belt prior to being transport to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175