# METHOD OF STATEMENT

Date: 26th Dec 2024

Description of work

To shift out 1 unit machine and accessories from production area to loading bay, hoist and load up onto wooden base for other transport to pack, after pack load up onto container

Machine Spec

1 unit Mazak Slant Turn 60N

Main machine: 970 x 304 x 330 cm / Wt: 28, 500 kg

Accessories: 292 x 135 x 152 cm / Wt: 700 kg

Accessories: 139 x 170 x 109 cm / Wt: 300 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

06/01/25 – shift out accessories to loading area for other transport to pack

06/01/25 – jack up machine to standby for next day moving

07/01/25 – to shift out machine from production to loading area, hoist up onto wooden base for packing, after packing load up onto container, secured with chocking wood and lashing belt

Time of Operation

09:45 am

Location of Operation

#### 273 Jalan Ahmad Ibrahim

#### Baker Hughes Singapore Pte

Equipment Use

1 x 100ton mobile crane

Lorry loader

Hydraulic jack

Woodblock

Rollers/electric rollers

Wire/webbing sling

Shackle

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacle on operation area and route to machine position

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile crane at designated hoisting area

- Setting up of crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Jack up machine to 4” high and let it sit on solid woodblock

- Insert rollers under machine bed

- Shift out machine from production to loading area

- Riggers to attached required sling lifting to crane hook

- Secure sling to rigging point of machine

- Once rigging work is done, signalman to give signal to crane operator to hoist up machine

- Load up machine onto wooden base for other transport to pack

- After pack, to load up case onto container

- Secured case with chocking wood and lashing belt

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175