# METHOD OF STATEMENT

Date: 21st May 2025

Description of work

To shift out 2 units machine and accessories from production area to loading bay, hoist up onto lorry chassis and deliver to scrapyard for disposal

Machine Spec

1 unit Waida Jig Grinding

Dim: 225 x 190 x 270 cm

Wt: 3, 500 kg

1 unit Matsuura Horizontal Mill

Dim: 360 x 240 x 250 cm

Wt: 8, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

03/06/25

Time of Operation

09:45 am

Location of Operation

#### 285 Jalan Ahmad Ibrahim

Panasonic Factory Solutions Asia Pacific

Equipment Use

2-3 x 633 lorry loader

Hydraulic/hand jack

Rollers/pallet truck

Wire sling

Webbing sling

Shackle

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting location

- Set up crane with outrigger fully extended and resting on steel plate/timber mats as base plate

- To jack up machine to 4” high

- Insert rollers under machine bed

- Shift out machine from production to loading area

- Riggers to insert required sling onto crane hook

- Secure sling to rigging point of machine

- Hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist and load up machine onto lorry chassis

- Released all sling from rigging point of machine

- Secured machine with strapping belt prior to moving out of customer premise

- Transport and deliver to scrapyard for disposal

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175