# METHOD OF STATEMENT

Date: 18th July 2022

Description of work

To shift out 2 unit machine from production to loading area, hoist up and load onto lorry chassis and deliver to another location

Machine Spec

1 unit Sino 60ton Power Press

Dim: 140 x 160 x 220 cm / Wt: 6, 000 kg

1 unit Dobby 45ton Hi Speed Press

Dim: 150 x 120 x 200 cm / Wt: 4, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

22/07/22

Time of Operation

09:45 am

Location of Operation

33 Gul Lane

Baumann Spring Company (S) Pte Ltd

Equipment Use

1-2 x 633 lorry loader

Rollers/pallet truck

1 set webbing sling

Shackle

Obstacles

No obstacle on crane and unloading area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to guide and control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine using hand jack

- Insert rollers/pallet truck pallet truck under machine bed

- Shift out machine from production to loading bay

- Attached sling to crane hook and secure onto rigging point of machine

- Hoist up and load machine onto lorry chassis

- Release all sling from rigging point of machine

- Secured machine with strapping belt prior to moving out of customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175