# METHOD OF STATEMENT

Date: 06th Jan 2022

Description of work

To shift out 1 unit machine to loading area, hoist up onto lorry chassis

Machine Spec

1 unit Precitech N200 Turning machine

Dim: 180 x 150 x 170 cm

Wt: 2, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

17/01/22

Time of Operation

09:45 am

Location of Operation

#### 50 Nanyang Ave, N3 B4C-03

Nanyang Technological University

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine using crowbar

- Insert pallet truck under machine bed

- Shift out machine to loading bay

- Attached required sling to crane hook and secured onto machine hoisting point

- Hoist up and load machine onto lorry chassis

- Secured machine with belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175