# METHOD OF STATEMENT

Date: 21st Aug 2018

Description of work

1) To shift out 5 unit Milling machine from workshop to loading area, hoist onto lorry chassis and transport to another building

2) To relocate 6 unit Milling, 1 unit Grinding and 3 unit work bench within W1218

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

23/08/2018   –   09:45 am

Location of Operation

#### 500 Dover Rd, Singapore Polytechnic

#### From Workshop W1412 to W1218

Equipment Use

1 x lorry crane

Pallet truck

Shackle/ wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine to 3” high

- Insert pallet truck to machine base

- Shift out machine from workshop to loading bay

- Hoist up and load machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of premise and deliver to new building

- At new location, unload machine from lorry chassis

- Transport machine into workshop

- Position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175