# METHOD OF STATEMENT

Date: 29th May 2017

Description of work

To shift out 1 unit CMM machine from production room to loading area, load onto lorry chassis and transfer to new building within same premises

Machine Spec

1 unit Global 575

Dim: 150 x 105 x 195 cm

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

31/05/2017 – Afternoon slot

Location of Operation

#### No 501 A.M.K Industrial Pk 1

Delphi Automotive Systems (S) Pte Ltd

Equipment Use

1 x lorry crane

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc.…safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Insert pallet truck to machine base

- Shift out machine from 3rd floor production room to loading bay via cargo lift

- Load machine onto lorry chassis via loading bay

- Secured machine with strapping belt

- Transport machine to new building within the same premise

- Hoist down machine from lorry chassis, shift and position into new premise at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175