# METHOD OF STATEMENT

Date: 11th June 2019

Description of work

To shift out 1 unit machine from production to loading bay, hoist up onto lorry chassis and deliver to new location for stuffing into container

Machine Spec

Makino EDNC machine

Dim: 240 x 210 x 250 cm

Wt: 5, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

14/06/2019

Time of Operation

10:00 am

Location of Operation

#### 51 Joo Koon Circle

OMNI Mold Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Hand jack/crowbar

1 set wire sling

Canvas/metal plate (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine to 3” using hand jack

- Use of woodblock to insert under machine bed

- Insert rollers/pallet truck under machine bed

- Shift out machine from production to loading bay

- Hoist up and load machine onto lorry chassis

- Secured machine with belt prior to moving out of premise

- Transport machine to other location and stuff into container

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175