# METHOD OF STATEMENT

Date: 03rd Sep 2019

Description of work

To shift out 1 unit CMM machine from 2nd floor to loading area via cargo lift, hoist up onto lorry chassis, secured with strapping belt and transport to new location

Machine Spec

1 unit 775 CMM machine

Dim: 700 x 700 x 500 mm

Wt: 750 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

10/09/2019

Time of Operation

Afternoon schedule

Location of Operation

#### No 62 Loyang Way

GE ATI

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set webbing sling

Canvas/plywood (for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

PPE Requirement

Safety helmet

Safety shoe

Safety glass

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E.

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Insert pallet truck onto machine stand

- Jack up pallet truck

- Shift out machine from 2nd floor production room to loading bay via cargo lift

- Unload machine from pallet truck

- Riggerman to secured webbing sling to machine lifting point

- Signalman will give signal to lorry loader operator to lift up machine slowly and check on balancing

- Lifting supervisor to check and confirm load are safe to be rig

- Hoist up and load machine onto lorry chassis

- Release all sling from machine

- Secured machine with strapping belt

- Transport machine out of premise and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175