# METHOD OF STATEMENT

Date: 15th July 2019

Description of work

To Hoist down 1 unit Machine Weight: 400kgs. Shift and position into premises at No.8 Tractor Road.

Machine Spec

1 unit Machine

Wt: 400kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

15/07/2019

Time of Operation

11:00 am

Location of Operation

8 Tractor Road

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Hand jack/crowbar

1 set wire sling

Canvas/metal plate (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Hoist Machine from Lorry Crane

- Load onto wooden block

- Insert rollers/pallet truck under machine bed

- Shift machine from hoisting area into premises

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175