# METHOD OF STATEMENT

Date: 30th May 2019

Description of work

To shift out 1 unit Generator from workshop to loading bay, hoist up onto lorry chassis and deliver to new location

Generator Spec

Dim: 130 x 130 x 220 cm

Wt: 6, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

03/06/2019

Time of Operation

10:00 am

Location of Operation

#### 8 Tuas Bay Lane

Equipment Use

1 x 633 lorry loader – PALFINGER PK50002EH

Rollers/pallet truck

1 set hand jack

1 set wire sling

1 set shackle

Canvas/metal plate (for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location

- Unload necessary equipment from lorry chassis

- Insert hand jack under generator bed

- Jack up generator to about 4”

- Insert rollers under generator bed

- Shift out generator from workshop to loading area

- Lorry loader to hoist up and load generator onto lorry chassis

- Secured generator with strapping belt prior to moving out of premise

- Deliver generator to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175