# METHOD OF STATEMENT

Date: 03rd Feb 2017

Description of work

To shift out 1 unit CMM machine from production room to loading area, hoist and load onto lorry chassis

Machine Spec

Chameleon 71007

Dim: 1.4 x 2.0 x 3.1 m

Wt: 2, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

06/02/2017 – 09:30 am

Location of Operation

#### No 23 Loyang Way

GE Aviation Service Operation LPP

Equipment Use

1 x lorry crane

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc.…safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Insert pallet truck to machine base

- Shift out machine from production room to loading bay

- Hoist up and load machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of customer premise and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175