# METHOD OF STATEMENT

Date: 25th May 2021

Description of work

To shift out 1 unit machine with bar feeder from production area to loading bay, hoist up onto lorry chassis and deliver to Eric Wong warehouse

Machine Spec

1 unit Miyano Auto Lathe

Dim: 280 x 158 x 184 cm

Wt: 2, 800 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

27/05/21

Time of Operation

09:30 am

Location of Operation

#### 16 Seletar Aerospace Crescent

JEP Precision Engineering Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Metal plate/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location

- Shift out machine from production to loading area

- Insert sling to machine hoisting point

- Hoist up machine about 1ft above ground to check on balancing

- After well balance, hoist up machine and load onto lorry chassis

- Secured machine with strapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175