# METHOD OF STATEMENT

Date: 28th June 2022

Description of work

To shift out 1 unit machine from production area to loading bay, hoist up and load onto low bed and secured with lashing belt and choking wood

Machine Spec

1 unit Litz CV1400 CNC

Dim: 385 x 335 x 310 cm

Wt: 15, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

05/07/22 (Tue)

Time of Operation

09:30 am

Location of Operation

#### 16 Seletar Aerospace Crescent

JEP Precision Engineering Pte Ltd

Equipment Use

1 x 633 lorry loader (FALFINGER PK110002SHH)

1 x 3 ton battery forklift

Rollers/pallet truck

Wire/webbing sling

Metal plate/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location

- To shift out machine from production to loading area

- Riggers to insert sling to crane hook and secured onto rigging point of machine

- Lorry loader to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up and load machine onto low bed

- Release all sling from rigging point of machine

- Cover up machine with canvas

- Secured machine with choking wood and lashing belt

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175