# METHOD OF STATEMENT

Date: 12th Nov 2024

Description of work

To shift out 2 unit machine and accessories from production area to loading bay, hoist up onto buyer arrange container, accessories and smaller item will be load up onto lorry and deliver to Eric Wong warehouse for stuffing into container

Machine Spec

2 unit E-1060V/6/8

Refer to attached machine spec

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

18-22/11/24

Time of Operation

10:00 am

Location of Operation

#### 16 Seletar Aerospace Crescent

JEP Precision Engineering Pte Ltd

Equipment Use

1 x 100 ton mobile crane

633 lorry crane

1 x 3 ton battery forklift

Hydraulic/hand jack

Rollers/pallet truck

Wire/webbing sling

Metal plate/canvas (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile/lorry crane at designated location

- Set up crane with outrigger fully extended and resting on steel plate/timber mats as base plate

- To jack up machine and insert rollers

- Shift out machine from production to loading area

- Riggers to attached lifting gear to crane hook machine

- Secure sling to rigging point of machine

- Hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist and load up onto buyer arrange container

- Released all sling from rigging point of machine

- Secured machine with lashing belt and choking wood

- Accessories and smaller item will be load up onto lorry and deliver to Eric Wong warehouse for stuffing into container

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175