# METHOD OF STATEMENT

Date: 21st July 2020

Description of work

To shift out 1 unit machine from production area to loading bay, assist engineer to dismantle machine into parts, after dismantle, load onto 2 x 40ft HQ container and secured with lashing belt and choking wood

Machine Spec

Youji YV1200 ATC – to dismantle into parts

Main body: 430 x 230 x 120 cm

Chip conveyor: 330 x 185 x 185 cm

Column with cross beam: 280 x 250 x 280 cm

Covers

Total wt: 22, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

28/07/20   -   Shift out from production and assist customer dismantle machine parts

29/07/20   -   Load machine into container and secured

Time of Operation

09:30 am

Location of Operation

#### 16 Seletar Aerospace Crescent

JEP Precision Engineering Pte Ltd

Equipment Use

1-2 x 633 lorry loader

1 x 3 ton battery forklift

Rollers/pallet truck

Wire/webbing sling

Metal plate/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location

- Shift out machine from production to loading area

- To assist customer engineer dismantle machine into parts

- After dismantle put aside

- On next day, hoist up machine and parts onto lorry chassis

- Reverse lorry crane to near container (back to back)

- Hoist up machine to near container door

- Lower down machine and insert rollers or metal rod under machine base

- Push and position machine into container

- Jack up and pull out rollers/metal rod

- Secured machine with choking wood and lashing belt

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd / Tel: 62824175