# METHOD OF STATEMENT

Date: 13th Apr 2020

Description of work

To hoist up 1 wooden case onto trailer/lorry chassis by using lorry loader

Case Spec

Dim: 445 x 260 x 300 cm

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

TBA

Location of Operation

15 Tuas View Place

John Crane Singapore Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

1 set wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E.

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area

- Shift out case to loading bay

- Lorry loader to hoist up case and load onto lorry/trailer chassis

- Secured machine with strapping belt

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175