# METHOD OF STATEMENT

Date: 15th July 2019

Description of work

To shift out 1 unit Global 9.12.8 CMM Machine

Machine Spec

1 unit Global 9.12.8 CMM Machine

(Dim: 250 x 160 x 195cm Wt: 3.5ton)

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

1/08/2019

Time of Operation

10:00 am

Location of Operation

No.4 Gul Way

Equipment Use

1 x Lorry crane

Rollers/pallet truck

Hand jack/crowbar

1 set wire sling

Canvas/metal plate

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack Machine up using Pallet Truck

- Load onto wooden block

- Insert rollers/pallet truck under machine bed

- Shift machine from hoisting area and packing area

- Lift machine onto wooden base

- Pack machine into crate

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175