# METHOD OF STATEMENT

Date: 20 Jan 2015

Description of work

To shift out 1 unit Eclipse CMM machine from production room to loading area, load onto wooden base for packing. After packing, load onto 1 x M’sia lorry

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

21/01/2016 – 09:30am

Location of Operation

#### No 18 Loyang Crescent

Pratt & Whitney Services Pte Ltd

Equipment Use

Lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc.…safety shoes, helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack/lift up machine to 4” high

- Insert rollers/pallet truck to machine base

- Shift out machine from production room to hoisting bay

- Load machine onto wooden pallet for packing

- After pack, hoist up and load machine onto M’sia lorry

- Secured machine with strapping belt prior to moving out of customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175