# METHOD OF STATEMENT

Date: 15th July 2020

Description of work

To shift out 2 unit machine from 4th floor to loading bay via cargo lift, hoist up onto lorry chassis and deliver to another location

Machine Spec

2 unit ACC350II Grinder

Dim: 1727 x 1526 x 1405 mm

Wt: 940 kg each

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

27/07/2020

Time of Operation

09:45 am

Location of Operation

#### 11 International Business Park

Hi-P International Limited

Equipment Use

1 x 633 lorry loader

Pallet truck

Crowbar/woodblock

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated loading/unloading area

- Shift out machine from production at 4th floor to loading area via cargo lift

- Attached sling to crane hook and secure to machine hoisting point

- Hoist up machine about 1ft above ground to check on balancing

- When well balance, hoist up machine and load onto lorry chassis

- Release all sling from machine

- Secured machine with strapping belt prior to moving out of premise

- Transport and deliver machine to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175