# METHOD OF STATEMENT

Date: 19th April 2016

Description of work

To shift out 1 unit E650H machine from existing location to loading area, hoist up onto low loader and deliver to new location

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

21/03/2016 – 10:00 am

Location of Operation

#### No 2 Gul Circle

#### Cameron (S) Pte Ltd

Equipment Use

1 x 100 ton mobile crane

Lorry crane

1 x forklift

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc.…safety shoes, safety glass, helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 3” high

- Insert rollers to machine base

- Shift out machine from production room to hoisting bay

- Hoist up and load machine onto low loader

- Low loader to transfer machine from existing new location

- Hoist down machine from low loader, insert rollers and transfer to new designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175