# METHOD OF STATEMENT

Date: 12th May 2022

Description of work

To shift out 2 case to loading area, load onto lorry chassis, transport to another location, hoist up to 3rd floor using overhead crane

Machine Spec

2 case of 1 set MJF 3D Printer

Dim: 2210 x 1200 x 1448 mm

Wt: 750 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

14/05/22

Time of Operation

09:45 am

Location of Operation

#### Collection: 10 Science Pk Rd #02-06

#### Deliver: 38F Tuas Ave 11

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

Any debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Shift out case to loading bay via cargo lift

- Hoist up and load onto lorry chassis

- Secured case with strapping belt

- Transport to new location

- Using customer overhead crane to lift up case to 3rd floor

- Transport into building premise

- Uncrate case

- Shift out machine from case using ramp

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175