# METHOD OF STATEMENT

Date: 07th Sep 2022

Description of work

To shift out 3 unit machine from production area to loading bay, hoist up and load onto low loader, transport to new location, unload, shift and position to location

Machine Spec

1 unit Makino FBNC Milling

Dim: 400 x 310 x 300 cm

Wt: 10, 000 kg

1 unit Starway VMC 1480 CNC

Dim: 350 x 320 x 290 cm

Wt: 10, 000 kg

1 case of CMM

Dim: 350 x 250 x 270 cm

Wt: 7, 800 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

08/09/22

Time of Operation

09:45 am

Location of Operation

#### Fong Lee Metal Industries Pte Ltd

#### Move out: 127 Pioneer Rd

Move in: 15 Pioneer Sector 3

Equipment Use

2 x 633 lorry crane

Low loader

Rollers/pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on operation area and route to machine position

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated hoisting area

- Shift out machine from production to loading area

- Attached required lifting gears to crane hook and secure to rigging point of machine

- Lorry crane to hoist up machine

- Reverse low loader to near machine

- Unload machine onto low loader chassis

- Release all sling from machine rigging point

- Secured machine with strapping belt

- Transport machine to new location

- Hoist down and unload machine from low loader

- Insert rollers onto machine base

- Shift and transport machine into production

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175