# METHOD OF STATEMENT

Date: 24th Sep 2020

Description of work

To shift out 2 unit machine from production to loading area, hoist up onto low-loader, transport to new factory, unload, shift and position into production at ground floor

Machine Spec

1 unit Leadwell LTC-50CXL CNC Lathe

Dim: 640 x 270 x 240 cm

Wt: 20, 000 kgs

1 unit Leadwell LTC-50C CNC Lathe

Dim: 480 x 245 x 240 cm

Wt: 13, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

25/09/20   -   09:45 am

Location of Operation

#### Fong Lee Metal Industries Pte Ltd

Move out – 127 Pioneer Rd

Move in – 15 Pioneer Sector 3

Equipment Use

1 x 50 ton mobile crane

1 x 633 lorry loader

1 x low-loader

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Shift out machine from production area to loading bay

- Hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up and load machine onto low-loader chassis

- Secured machine with strapping belt

- Transport machine to new factory

- At new factory, hoist down machine from low-loader chassis

- Insert rollers under machine base

- Shift and transport machine into production

- Position machine at the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175