# METHOD OF STATEMENT

Date: 09th June 2022

Description of work

To shift out 1 unit press machine from production area to loading bay, hoist up and load onto low loader, transport to new factory, unload, shift and position to location

Machine Spec

1 unit PK-40 press

Wt: 22, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

16-17/06/22

Time of Operation

09:45 am

Location of Operation

#### Shimano (S) Pte Ltd

#### Move out: 20 Benoi Sector

Move in: 11 Bulim Walk

Equipment Use

1 x 110ton lorry crane

Low loader

633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacle on operation area and route to machine position

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to guide and control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated hoisting area

- Shift out machine from production to loading area

- Attached required lifting gears to crane hook and secure to hoisting point of machine

- Lorry crane to hoist up machine

- Load up machine onto low loader chassis

- Release all sling from crane hook

- Secured machine with chain belt

- Transport machine to new factory

- Hoist down and unload machine from low loader

- Insert rollers onto machine base

- Shift and transport machine into production

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd / Tel: 62824175