# METHOD OF STATEMENT

Date: 16th Aug 2020

Description of work

To shift out 1 unit machine from production area to loading bay using factory overhead crane, load onto lorry chassis, transport to new location, unload, shift and position into production

Machine Spec

1 unit Kyori High Speed 60ton Press machine

Dim: 230 x 140 x 285 cm

Wt: 8, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

18/09/20   –   Afternoon schedule

Location of Operation

#### 20 Benoi Sector

Shimano (S) Pte Ltd

Equipment Use

1 x 633 lorry loader – PALFINGER PK100002E

Rollers/pallet truck

Wire/webbing sling

1 x platform (to lay out for step at new location)

1 x factory overhead crane

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated location

- Using overhead crane to hoist and shift out machine from production to loading area

- Load machine onto lorry chassis

- Secured machine with strapping belt

- Transport machine to new location

- Lorry loader to hoist down machine from lorry chassis

- Insert rollers onto machine base

- Shift and transport machine into production

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175