# METHOD OF STATEMENT

Date: 29th Apr 2019

Description of work

To shift out 1 unit machine from production area to loading bay, hoist up onto lorry chassis, transport to new location, unload, shift and position into production

Machine Spec

1 unit Bullard 42 VTL

Dim: 300 x 280 x 310 cm

Wt: 10, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

06/05/2019   -   09:30 am

Location of Operation

#### From 21 Loyang Lane   –   23 Loyang Lane

Windsor Airmotive Asia Pte Ltd

Equipment Use

1 x 633 lorry loader – PALFINGER PK100002E

Rollers/pallet truck

Wire/webbing sling

Metal plate/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated location

- Shift out machines from production to loading area

- Hoist up and load machine onto lorry chassis

- Secured machine with strapping belt

- Transport machine to new location

- Hoist down machine from lorry chassis

- Insert rollers onto machine base

- Shift and transport machine into production

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175