# METHOD OF STATEMENT

Date: 23rd Nov 2022

Description of work

To shift out 1 unit machine from ground floor to loading area, hoist up onto lorry chassis, transport to another location, unload, shift and position to the designated location

Machine Spec

1 unit Mazak Integrex 200

Dim: 400 x 225 x 255 cm

Wt: 10, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

30/11/22

Time of Operation

09:45 am

Location of Operation

#### 500 Dover Rd, Singapore Polytechnic

#### From W1411   –   W12

Equipment Use

1 x 633 lorry loader

1 x platform (to lay out for machine pathway)

Rollers/pallet truck

Wire/webbing sling

Canvas/metal sheet (to lay out for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

Any debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Shift out machine from ground floor to loading area

- Attached required sling to crane hook and secured to rigging point of machine

- Hoist up machine and load onto lorry chassis

- Release all sling from rigging point of machine

- Secured machine with strapping belt

- Transport to new location

- Hoist down machine from lorry chassis to near ground level

- Insert rollers under machine bed

- Transport machine into building premise at ground floor

- Position machine at temporary storage area

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175