# METHOD OF STATEMENT

Date: 19th July 2021

Description of work

To shift out 1 unit machine from 4th floor to loading area, hoist up onto lorry chassis, transport to another location, unload, shift and position to the designated location

Equipment Spec

1 unit Table Top Milling machine

Wt: 360 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

22/07/21

Time of Operation

09:45 am

Location of Operation

#### 500 Dover Rd, Singapore Polytechnic

#### Block T14 – Block W12

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

Any debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Shift out machine from 4th floor to loading bay via cargo lift

- Attached required sling to crane hook and secured to load to be lifted

- Hoist up machine and load onto lorry chassis

- Secured machine with strapping belt

- Transport to new location

- Hoist down machine from lorry chassis to the ground

- Using pallet truck to transport machine into building premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175