# METHOD OF STATEMENT

Date: 12th Apr 2024

Description of work

To shift out 1 unit machine from production area to loading bay, hoist up and load onto lorry chassis, transport to another factory, unload, shift and position to location

Machine Spec

Wenzel LH1512 CMM machine

Refer to attached

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

17/04/24

Time of Operation

09:45 am

Location of Operation

#### Avitron Pte Ltd

#### Move out: 8 Tuas Ave 20

#### Move in: 51 Corporation Rd

Equipment Use

2 x 633 lorry crane

Rollers/pallet truck

Wire/webbing sling

Shackle

Hydraulic/hand jack

Woodblock

Obstacles

No obstacle on operation area and route to machine position

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to guide and control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated hoisting area

- Shift out machine from production to loading area

- Riggers to attached required sling to crane hook

- Secure sling to rigging point of machine

- Lorry loader to hoist up machine

- Load up machine onto lorry chassis

- Release all sling from crane hook

- Secured machine with strapping belt

- Transport machine to another factory

- Hoist down and unload machine from lorry chassis

- Insert rollers onto machine bed

- Shift and transport machine into production

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175