# METHOD OF STATEMENT

Date: 09th July 2019

Description of work

To shift out 2 unit machine from workshop at ground floor to loading area, hoist up onto lorry chassis, transport and deliver to our warehouse for temporary storage,

Machine Spec

1 unit Hurco VMX30

Dim: 240 x 190 x 220 cm

Wt: 3, 000 kgs

1 unit Hermle B300U

Dim: 260 x 240 x 260 cm

Wt: 8, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

13/07/2019

Time Of Operation

09:45 am

Location of Operation

#### Nanyang Technological University (NTU)

#### 81 Nanyang Drive, School of Art, Design & Media

Equipment Use

2 x lorry crane

Pallet truck/rollers

Wire/webbing sling

1 x forklift

Canvas, plywood, metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated location

- Lay out canvas, plywood and metal sheet for floor protection

- Shift out machines from workshop to loading area

- Hoist up and load machine onto lorry chassis

- Secured machine with strapping belt

- Transport and deliver machine to warehouse for temporary storage

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175