# METHOD OF STATEMENT

Date: 14th Dec 2021

Description of work

To shift out 3 unit machine to loading area, hoist up onto lorry chassis, transport to another location, unload, shift and position to the designated location

Equipment Spec

1 unit Metacut 251 machine

Dim: 71 x 70 x 54 cm / Wt: 126 kg

2 unit Grinder/Polisher / Wt: 30 kg each

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

16/12/21

Time of Operation

09:45 am

Location of Operation

#### 9 Woodlands Ave 9, Republic Polytechnic

#### Collect: Block E4, Level 1, Room L

#### Deliver: W33, Block W5, Level 1, Room C

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

Any debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Shift out machine to loading bay

- Hoist up machine and load onto lorry chassis

- Secured machine with strapping belt

- Transport to new location

- Hoist down machine from lorry chassis to the ground

- Using pallet truck to transport machine into building premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175