# METHOD OF STATEMENT

Date: 09th July 2020

Scope of work

To hoist out 1 set Oven through rooftop opening by using mobile crane, load onto low-loader/lorry chassis and deliver

Oven spec – to be dismantle by customer engineer

Unwinding

Dim: 325 x 145 x

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

27/06/2020   -   09:00 am

Location of Operation

#### Heng Long Leather Co. Pte Ltd

#### Collection: 50 Defu Lane 7

Deliver: 6 Kim Chuan Drive

Equipment Use

1-2 x 633 lorry loader

Rollers/pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Shift out machine from production to unloading area

- Hoist up and load machine onto lorry chassis

- Secured machine with strapping belt

- Transport machine to new factory

- At new factory, hoist down machine from lorry chassis

- Insert rollers/pallet truck to machine base

- Shift and transport machine into factory premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175