# METHOD OF STATEMENT

Date: 01st Apr 2022

Description of work

To shift out crate from 3rd floor via cargo lift, load onto lorry chassis, transport to another location, unload, uncrate, shift and position to the designated location

Spec

1 unit machine

Wt: 259 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

08/04/22 (Friday)

Time of Operation

09:45 am

Location of Operation

#### Collection: NTU, 50 Nanyang Drive, Research Techno Plaza #03-01

#### Deliver: NTU, 50 Nanyang Ave, Blk S1-B3a-01

Equipment Use

1 x 633 lorry loader

1 x stacker

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

Any debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Shift out crate to loading bay via cargo lift

- Hoist up and load onto lorry chassis

- Secured crate with strapping belt

- Transport to new location

- Hoist down crate from lorry chassis to the ground

- Uncrate case

- Using pallet truck to transport into building premise

- Position at the designated location

- Using stacker to load onto table

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175