# METHOD OF STATEMENT

Date: 03rd Apr 2020

Description of work

To shift out 1 unit machine from workshop to loading area, hoist up onto lorry chassis, transport to another location, unload, shift and position into premise at ground floor

Machine Spec

1 unit Washing Line

Dim: 180 x 90 x 200 cm

Wt: 900 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

06/04/2020

Time of Operation

Afternoon schedule

Location of Operation

#### Collection: 70A Nanyang Dr, Blk HW2

#### Deliver: Blk N4

Equipment Use

1 x 633 lorry loader

Pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated loading area

- Shift out machines from workshop to loading area

- Hoist up and load machine onto lorry chassis

- Secured machine with strapping belt

- Transport machine to new location

- Hoist down machine from lorry chassis to the ground

- Insert pallet truck onto machine base

- Shift machine into premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175